1.List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

| Hardware issue | Typical symptom |
| --- | --- |
| Faulty RAM | PC may fail to POST, produce beep codes, or show a blank display. |
| Failing power supply (PSU) | PC may not start, restart unexpectedly, or shut down under load. |
| Overheating CPU | PC may become slow, freeze, shut down, or restart unexpectedly. |
| Faulty hard drive/SSD | Very slow booting, operating-system errors, crashes, or inability to boot. |
| Faulty graphics card | No display, graphical artifacts, crashes, or distorted images. |

2. Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

- Check the wall outlet
Plug another known-working device into the outlet to make sure AC power is available.

- Inspect the charger and cable
Look for cuts, bent connectors, exposed wires, or burn marks. Gently check whether the cable or plug feels unusually loose.

- Check the charging connector
Inspect the laptop's charging port for dust, bent contacts, or physical damage. Make sure the plug fits securely.

- Test the charger output with a multimeter
Set the multimeter to DC voltage (V⎓). Place the probes across the charger's output contacts, observing the correct polarity. Compare the measured voltage with the rating printed on the charger's label.

- Check the battery connection
If the laptop has an accessible removable battery, remove it and inspect its contacts for dirt or damage. Reseat it firmly.

- Try charging without the battery, if the design allows it
If the laptop operates from the adapter but not with the battery installed, the battery may be faulty.

- Check for physical motherboard/charging-circuit damage
If the adapter produces its rated voltage and the connector/battery appear sound, the laptop's charging circuit or motherboard may require repair.

3.Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

For a standard USB 5 V charger, the expected output is approximately 4.75–5.25 V DC.

Procedure:

- Set the multimeter to DC voltage (V⎓), preferably a 20 V range if it is not autoranging.

- Plug the USB charger into the wall.

- Connect the USB cable to the charger.

- Touch the black probe to GND (USB negative) and the red probe to the +5 V/VBUS contact.

- Record the displayed voltage.

A suitable example record for a classroom exercise would be:

Measured voltage: 5.02 V DC
Expected range: 4.75–5.25 V

4. A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

Steps

- Shut down the PC and unplug the AC power cord.

- Disconnect the ATX power connector from the motherboard.

- Set the multimeter to DC voltage (V⎓).

- Turn the PSU on only if your test setup safely permits it; follow the PSU/motherboard manufacturer's procedure for powering the disconnected connector.

- Locate a yellow wire (+12 V) and a black wire (ground) on the ATX connector.

- Place the black probe on the black wire.

- Place the red probe on the yellow wire.

- Read and record the voltage.

Example measurement:

Measured 12 V rail: 12.08 V DC
Expected nominal voltage: approximately 12 V