1.List 5 common BIOS or POST error messages you might see when starting a PC and write what hardware or configuration issue each one usually points to.

1. Five common BIOS/POST error messages

| BIOS/POST message | Hardware/configuration issue it usually indicates |
| --- | --- |
| CMOS checksum error – Defaults loaded | CMOS/BIOS settings have become invalid, often because of a weak CMOS battery or corrupted settings. |
| Keyboard error or no keyboard present | Keyboard is disconnected, faulty, or the keyboard/controller connection has a problem. |
| CMOS battery low | The motherboard's CMOS/RTC battery is losing its charge and may no longer retain BIOS settings. |
| Disk boot failure / No boot device found | The storage drive is not detected, is faulty, or the boot device/order is configured incorrectly. |
| CPU fan error | The BIOS cannot detect the CPU cooling fan, or the fan is disconnected/failing. |

2.Given this scenario: On boot, a PC beeps 3 times in a repeating pattern and does not display anything on the screen. Research and write what this POST beep code typically means for an AMI BIOS, and suggest two troubleshooting steps.

For traditional AMI BIOS, 3 short beeps typically indicate a base-memory failure, historically described as a failure in the first 64 KB of system memory. Modern motherboard implementations can vary, so the specific motherboard manual should be checked as well.

Two troubleshooting steps:

- Power off and reseat the RAM: Disconnect power, remove the RAM modules, check the contacts/slots, and firmly reinstall the modules.

- Test RAM individually: If multiple RAM sticks are installed, try booting with one module at a time and, if necessary, try another memory slot or a known-good compatible module.

3.Access the BIOS setup utility on any available PC or virtual machine, take a screenshot of the main BIOS screen, and identify the motherboard manufacturer and BIOS version.

This part needs to be completed on an actual available PC or virtual machine, because the motherboard manufacturer and BIOS version depend on the machine being examined.

Steps

- Restart the PC.

- During startup, repeatedly press the BIOS/UEFI setup key—commonly Delete, F2, F10, or Esc, depending on the manufacturer.

- On the main BIOS/UEFI screen, locate:

  - Motherboard/System manufacturer

  - BIOS/UEFI version

- Take a screenshot of the main screen.

- Crop out usernames, serial numbers, asset tags, or other personal information before submitting.

4.You are given a screenshot of a POST error message that says 'CMOS checksum error – Defaults loaded.' Explain what this means and describe how you would resolve this issue in a real system.<br><br><em><strong>Hint:</strong> Think about the CMOS battery and BIOS settings reset.</em>

This message means that the BIOS detected that the stored CMOS configuration does not match the expected checksum. As a precaution, the BIOS has discarded or ignored the saved settings and loaded its default configuration. A depleted CMOS battery is a common cause, although corrupted settings or other motherboard/firmware problems can also cause it. AMI documentation describes CMOS validity checking and loading default CMOS settings during POST. DDOS Days

How to resolve it

- Enter the BIOS/UEFI setup.

- Check the date and time and correct them if necessary.

- Reconfigure any required BIOS settings, such as boot order or memory settings.

- Save the settings and restart the PC.

- If the error returns after every shutdown, replace the CMOS/RTC battery with the correct compatible battery.

- Enter BIOS again, load/reconfigure the desired settings, save them, and restart.

- If the error continues despite a new battery, investigate possible CMOS/BIOS corruption or motherboard hardware problems.