1.Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

Steps

- Shut down the computer completely and disconnect the power cable/charger.

- Press the power button for a few seconds to discharge residual power.

- Open the computer case or the laptop's RAM access panel.

- Locate the RAM module(s) in their memory slots.

- Release the retaining clips on both ends of the RAM slot.

- Carefully remove the RAM module by holding its edges—avoid touching the gold contacts.

- Inspect the module and slot for dust, dirt, or physical damage.

- Align the notch on the RAM with the key in the slot.

- Press the module firmly and evenly until the retaining clips lock into place.

- Reconnect power and start the computer.

- Confirm that the computer boots normally and recognizes the installed memory.

2. Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

- 1. Identify the symptom — Determine whether there is no display, graphical artifacts, driver crashes, poor performance, or crashes during games.

- 2. Check the monitor and cable — Confirm the monitor is powered on, the correct input is selected, and the video cable is firmly connected.

- 3. Check the GPU connection — Power down the PC and make sure the graphics card is properly seated in its PCIe slot.

- 4. Check GPU power connectors — Verify that all required PCIe power connectors are securely attached to the graphics card.

- 5. Check display output — Make sure the monitor cable is connected to the GPU's video output, rather than the motherboard output when a discrete GPU is being used.

- 6. Check drivers — In the operating system, check whether the GPU is detected correctly and install/update the appropriate graphics driver if necessary.

- 7. Check temperatures — Look for excessive GPU temperatures or abnormal fan behavior, particularly when gaming.

- 8. Test with a known-good component — If possible, test the PC with a known-good GPU, cable, or monitor to isolate the faulty component.

- 9. Confirm the repair — Run a game or graphics stress test and verify that the original display problem, artifacts, or crashes no longer occur.

3.Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

SATA SSD/HDD

- Shut down the PC and disconnect AC power.

- Disconnect the SATA data cable or SATA power cable from the drive.

- Start the computer and enter BIOS/UEFI.

- Check the Storage/SATA Information or Boot section.

- If the drive is disconnected, it should normally disappear from the list of detected storage devices.

- Reconnect the SATA cable securely.

- Restart and enter BIOS again.

- Confirm that the drive is detected.

- Boot into the operating system and verify that the drive is accessible.

4.Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

Of the three choices given, I would check the RAM first. Memory problems can cause unpredictable crashes, freezes, and restarts, and RAM is relatively straightforward to isolate by reseating and testing modules individually. This is a troubleshooting starting point, not proof that RAM is the cause.

RAM diagnostic procedure

- Record the symptoms: Note when the restart occurs—for example, during gaming, immediately after startup, or while idle.

- Power off the PC: Disconnect the power before touching the RAM.

- Reseat the RAM: Remove the modules and reinstall them firmly.

- Check for physical problems: Inspect the RAM and slots for obvious damage or contamination.

- Test one module at a time: If two or more modules are installed, test each module individually in the recommended slot.

- Try another slot: If a particular module works in one slot but not another, the slot or motherboard may be involved.

- Check whether the restarts continue: Use the computer under the same conditions that previously caused the problem.

- Confirm the result: If the restarts stop after identifying/replacing a faulty RAM module, the RAM is a strong candidate for the cause.