1.A user reports their laptop won't power on even when plugged in. List 5 troubleshooting steps you would take to diagnose the cause, and for each step, explain what you are checking for.

| Step | What to do | What you are checking |
| --- | --- | --- |
| 1 | Check the wall outlet and charger | Confirms that AC power is available and the adapter is connected correctly. |
| 2 | Inspect the charger and charging port | Looks for damaged cables, bent pins, loose connectors, or physical damage. |
| 3 | Check the charging/power indicator | Determines whether the laptop is receiving power from the adapter. |
| 4 | Perform a power reset | Disconnect the charger, remove the battery if removable, hold the power button for about 15–30 seconds, then reconnect power and try again. This can clear a temporary power-state problem. |
| 5 | Test the charger/battery | Use a compatible known-good charger or, where appropriate, a multimeter to check the adapter's DC output. This helps distinguish a faulty charger/battery from a motherboard or power-circuit fault. |

2. You receive a helpdesk ticket: 'My monitor is showing a blank screen, but the CPU seems to be running.' Write a step-by-step troubleshooting flow you would follow to identify and resolve the issue.<br><br><em><strong>Hint:</strong> Consider checking both hardware connections and basic display settings.</em>

Troubleshooting flow

Start → Blank screen

- Check whether the monitor is powered on

  - Confirm the power LED is illuminated.

  - Check the power cable and wall outlet.

  - Try another known-good outlet if necessary.

- Check the monitor input

  - Use the monitor's buttons/menu to make sure the correct input is selected—for example, HDMI, DisplayPort, or VGA.

- Check the video cable

  - Disconnect and firmly reconnect the cable at both ends.

  - Inspect it for damage.

  - If possible, test with a known-good cable.

- Check where the cable is connected

  - If the PC has a dedicated graphics card, make sure the monitor is connected to the graphics card's output, rather than an unused motherboard video output.

- Test the monitor

  - Connect the monitor to another working computer.

  - If it works there, the monitor is probably not the source of the problem.

- Check the PC's display hardware

  - Confirm that the graphics card is seated properly and its required power connectors are attached.

  - If there is no dedicated GPU, check the motherboard's display connection and hardware.

- Check basic display settings

  - If Windows is running but the screen is blank, try Win + Ctrl + Shift + B to reset the graphics driver.

  - Use Win + P to cycle through display modes if the system is outputting to the wrong display.

- Test with another monitor/display

  - If another display works, the original monitor or cable is likely involved.

  - If neither display works, investigate the GPU, RAM, motherboard, or other PC hardware.

3.A user complains that their wireless mouse is unresponsive, but the keyboard works fine. Describe how you would isolate whether the issue is with the mouse, the USB port, or the system settings.

- Check the mouse itself

  - Replace or recharge its batteries.

  - Turn the mouse off and back on.

  - Check whether its optical sensor/light is operating.

  - If possible, test the mouse on another computer.

- Check the wireless receiver

  - If it uses a USB receiver, remove it and reconnect it.

  - Try the receiver in a different USB port.

  - Avoid using a faulty or loose USB hub while testing.

- Test the USB port

  - Plug another known-working USB device into the same port.

  - If the other device also fails, the USB port or its connection may be the problem.

- Check system settings

  - Verify that the operating system detects the mouse/receiver.

  - Check Device Manager for hardware errors.

  - If Bluetooth is used instead of a USB receiver, verify that Bluetooth is enabled and the mouse is still paired.

- Compare results

  - Mouse fails on multiple computers: likely mouse/battery problem.

  - Mouse works elsewhere but receiver fails in one USB port: likely USB-port/system issue.

  - Mouse and receiver work on another computer but not this one: investigate drivers, Bluetooth/USB settings, or the operating system.

4.Imagine you are working on a helpdesk team for a gaming cafe. One system is overheating and shutting down during use. List 3 possible hardware causes and describe how you would check each one.<br><br><em><strong>Hint:</strong> Think about fans, dust, and thermal paste.</em>

1. Failed or slow cooling fans

Possible cause: A CPU, GPU, or case fan may have stopped working or be spinning too slowly.

How to check:

- Visually inspect the fans while the PC is running.

- Listen for unusual noises.

- Check whether the CPU/GPU cooler fans spin when the system heats up.

- A failed fan should be repaired or replaced.

2. Dust blocking airflow

Possible cause: Dust buildup on heatsinks, fans, and air filters can restrict airflow and cause temperatures to rise.

How to check:

- Power down and disconnect the PC.

- Inspect the CPU/GPU heatsinks, case filters, and fan blades.

- Clean accumulated dust using appropriate compressed-air cleaning procedures.

- Make sure the intake and exhaust vents aren't blocked.

3. Dried or poorly applied thermal paste

Possible cause: Old or improperly applied thermal compound can reduce heat transfer between the CPU/GPU and its heatsink.

How to check:

- If temperatures remain unusually high after confirming that the fans and airflow are good, inspect the cooler mounting.

- If appropriate for the hardware and technician, remove the cooler, clean the old thermal compound, apply fresh compatible thermal paste, and reinstall the cooler evenly.

- Re-test the system under the same gaming workload to confirm that temperatures have improved.